# UHSLC Tidal Datums and Predictions

## Purpose:

Update datums and tide predictions for all Fast Delivery and Research Quality hourly data that is served via ERDDAP.

## Protocol

Learn best practices from legacy software, and make improvements where necessary. The deliverable is a Python script that may be ran in batch mode to cover groups of stations (all, FD, RQ, subsets, or individual stations).

### Output files

Netcdf

One file per “station” record (e.g., 001, 001a, 001b, 001c would cover Pohnpei for FD and its three RQ versions)

### Reference frames and units

Station zero

GMT

mm integer

### Epochs and data completion

These are the primary epochs: NTDE_1983-2001, NTDE_2002-2020, IPCC-AR6_1995-2014.

Attempt to calculate the datums and tidal harmonics/predictions for each of the primary 19-year epochs. However, this will be impossible for many “station” records, because of their data coverage. Use a 75% of hourly data for a completion criteria. If there is not 75% coverage for a particular epoch, skip it. For all records with at least 6 months of hourly data (>75% complete), make sure to define at least one epoch. That one epoch should be the most recent data, not to exceed 19 years. No station should have more than three epochs. Example: there will be one epoch for 001a (RQ), which will span 1969-05-09 to 1971-02-27.

### Datums
Note that since you do not have access to the switch elevations, it is okay to omit that. Otherwise, learn from the legacy software what datums to calculate and save.

### Tidal harmonics

Harmonics should be calculated for each epoch. Include the tidal harmonics in the output file. Follow best practices learned from the legacy software, especially concerning treatment of the nodal cycle and trends.

### Tide predictions

Tide predictions should be calculated for each set of harmonics. For FD stations: Output hourly predictions spanning the epoch start to 2100-12-31 23 hour. Also calculate minute predictions from each epoch end to 2030-12-31 23 hour (FD stations only). To reduce file sizes, only the times and heights of high/low daily tides from the minute predictions should be saved (review the legacy procedure). For the RQ stations: only calculate and save the hourly tide predictions spanning the available epochs (e.g., the prediction for 001a will cover the hours spanning 1969-05-09 to 1971-02-27).

### Test stations 
Run tests on generate plots for the following stations, FD and all RQ versions: 001, 002, 003, and 007 (datums, hourly predictions/residuals, and minute predictions if application).

## Legacy software

### Datums

%%==================================================================

%% Tidal Datums

%  https://tidesandcurrents.noaa.gov/datum_options.html

for station_ct = 1:size(uhidTxt,1)

if ~isnan(STATION(station_ct).latitude{1})

%% Generated by SEA to more closely match NOAA's method using a tidal day (MJW: 7/14/2025)

disp(strcat('Calculating Tidal Datums (Tidal Day Window): ',STATION(station_ct).station_UHSLC));

sl_NTDE = STATION(station_ct).sl_NTDE(:,1);

t_NTDE = STATION(station_ct).t_NTDE(:,1);

% Remove NaNs

valid_idx = ~isnan(sl_NTDE) & ~isnan(t_NTDE);

sl_NTDE = sl_NTDE(valid_idx);

t_NTDE = t_NTDE(valid_idx);

if numel(sl_NTDE) > t_min

%% --- MHW (Mean High Water) and MLW (Mean Low Water) using all local maxima/minima ---

[~, locs_high] = findpeaks(sl_NTDE, 'MinPeakDistance', 6); % Find all local maxima (high waters, at least 6 hours between peaks)

[~, locs_low]  = findpeaks(-sl_NTDE, 'MinPeakDistance', 6); % Find all local minima (low waters, at least 6 hours between troughs)

HW = sl_NTDE(locs_high);

LW = sl_NTDE(locs_low);

MHW_NTDE = mean(HW);

MLW_NTDE = mean(LW);

STATION(station_ct).MHW_NTDE = MHW_NTDE;

STATION(station_ct).MLW_NTDE = MLW_NTDE;

%% --- TIDETYPE: Diurnal or Mixed/Semidiurnal (using high tide intervals) ---

t_highs = t_NTDE(locs_high); % Use high waters calculated above

t_delta = diff(t_highs); % In days (assuming t_NTDE is in days)

if nanmean(t_delta) > 0.75

diurnal_flag = 1;

STATION(station_ct).TideType = 'Diurnal';

else

diurnal_flag = 0;

STATION(station_ct).TideType = 'Semidiurnal/Mixed';

end

%% --- MHHW, MLLW, MSL using running tidal day window (24h50m = 1.0347 days) ---

tidal_day_days = (24 + 50/60) / 24;

t_start = t_NTDE(1);

t_end = t_NTDE(end);

%window_starts = t_start : 1 : (t_end - tidal_day_days);

window_starts = t_start : tidal_day_days : (t_end - tidal_day_days); % Avoids overlapping bins

n_windows = length(window_starts);

max_list = nan(n_windows,1);

min_list = nan(n_windows,1);

mean_list = nan(n_windows,1);

for i = 1:n_windows

t0 = window_starts(i);

t1 = t0 + tidal_day_days;

idx = t_NTDE >= t0 & t_NTDE < t1;

window_data = sl_NTDE(idx);

if sum(~isnan(window_data)) >= 20 % Checks if there are at least 20 valid (non-NaN) data points in the window.

max_list(i) = max(window_data);

min_list(i) = min(window_data);

mean_list(i) = mean(window_data);

end

end

% Remove NaNs

max_list = max_list(~isnan(max_list));

min_list = min_list(~isnan(min_list));

mean_list = mean_list(~isnan(mean_list));

% Calculate datums

MHHW_NTDE = mean(max_list);

MLLW_NTDE = mean(min_list);

%MSL_NTDE  = mean(mean_list); % Calculated below as simple average of the NTDE time series

STATION(station_ct).MHHW_NTDE = MHHW_NTDE;

STATION(station_ct).MLLW_NTDE = MLLW_NTDE;

%STATION(station_ct).MSL_NTDE  = MSL_NTDE; % Calculated below as simple average of the NTDE time series

% -------------------------------------------

% Save timeseries referenced to MLLW datums:

sl_NTDE_MLLW = sl_NTDE - MLLW_NTDE;

% Display datums

disp(['MHHW (tidal day window): ', num2str(MHHW_NTDE), ' m']);

disp(['MLLW (tidal day window): ', num2str(MLLW_NTDE), ' m']);

%disp(['MSL  (tidal day window): ', num2str(MSL_NTDE),  ' m']);

disp(['MHW  (all highs): ', num2str(MHW_NTDE),  ' m']);

disp(['MLW  (all lows): ', num2str(MLW_NTDE),  ' m']);

% -------------------------------------------

% Higher High Water (HHW) and Lower Low Water (LLW) timeseries using tidal day windows

% max_list and min_list are already calculated above as the max/min in each tidal day window

STATION(station_ct).HHW_NTDE_MLLW = max_list - MLLW_NTDE; % Timeseries of HHW referenced to MLLW

STATION(station_ct).LLW_NTDE_MLLW = min_list - MLLW_NTDE; % Timeseries of LLW referenced to MLLW

clear max_list min_list MLLW_NTDE

%% -------------------------------------------

%% Mean Diurnal High/Low Water Inequality (Revised 11/14/2022)

% One-half the average difference between the two high/low waters of each tidal day observed over the National Tidal Datum Epoch.

% It is obtained by subtracting the mean of all the high waters from the mean of the higher high waters.

STATION(station_ct).DHQ_NTDE = STATION(station_ct).MHHW_NTDE - STATION(station_ct).MHW_NTDE;

% It is obtained by subtracting the mean of the lower low waters from the mean of all the low waters.

STATION(station_ct).DLQ_NTDE =  STATION(station_ct).MLW_NTDE - STATION(station_ct).MLLW_NTDE;

%% -------------------------------------------

%% Diurnal Tide Level (DTL)

DTL_NTDE = (STATION(station_ct).MHHW_NTDE + STATION(station_ct).MLLW_NTDE)/2;

STATION(station_ct).DTL_NTDE = DTL_NTDE;

clear('DTL_NTDE');

%% -------------------------------------------

%% Mean Tide Level (MTL)

MTL_NTDE = (STATION(station_ct).MHW_NTDE + STATION(station_ct).MLW_NTDE)/2;

STATION(station_ct).MTL_NTDE = MTL_NTDE;

clear('MTL_NTDE');

%% -------------------------------------------

%% Mean Sea Level (MSL)

MSL_NTDE = nanmean(sl_NTDE,1);

STATION(station_ct).MSL_NTDE = MSL_NTDE;

clear('MSL_NTDE');

%% -------------------------------------------

%% Great Diurnal Range (GT)

GT_NTDE = STATION(station_ct).MHHW_NTDE - STATION(station_ct).MLLW_NTDE;

STATION(station_ct).GT_NTDE = GT_NTDE;

clear('GT_NTDE');

%% -------------------------------------------

%% Mean Range of Tide (MN)

MN_NTDE = STATION(station_ct).MHW_NTDE - STATION(station_ct).MLW_NTDE;

STATION(station_ct).MN_NTDE = MN_NTDE;

clear('MN_NTDE');

%% -------------------------------------------

%% Greenwich High Water Interval (HWI)

% The average interval (in hours) between the moon's transit over the Greenwich meridian and the following high water at a location.

%  incomplete...

STATION(station_ct).HWI_NTDE = NaN;

%% -------------------------------------------

%% Greenwich Low Water Interval (LWI)

% The average interval (in hours) between the moon's transit over the Greenwich meridian and the following low water at a location.

%  incomplete...

STATION(station_ct).LWI_NTDE = NaN;

%% -------------------------------------------

%% Extremes (90%, 95%, 99%)

thresholds = [100-90, 100-95, 100-99];

thresholds_txt = {'90','95','99'};

for thres_ct = 1:size(thresholds,2)

%% Hourly

tmp_extrmLow = prctile(sl_NTDE_MLLW(:,1),thresholds(thres_ct));

tmp_extrmHigh = prctile(sl_NTDE_MLLW(:,1),100-thresholds(thres_ct));

name_extrmLow = ['SLntde_extrmLow_',thresholds_txt{thres_ct}];

name_extrmHigh = ['SLntde_extrmHigh_',thresholds_txt{thres_ct}];

STATION(station_ct).(name_extrmLow) = tmp_extrmLow;

STATION(station_ct).(name_extrmHigh) = tmp_extrmHigh;

clear tmp_extrmLow tmp_extrmHigh name_extrmLow name_extrmHigh

%% Daily (Lower Low Water and Higher High Water)

tmp_extrmLow = prctile(STATION(station_ct).LLW_NTDE_MLLW(:,1),thresholds(thres_ct));

tmp_extrmHigh = prctile(STATION(station_ct).HHW_NTDE_MLLW(:,1),100-thresholds(thres_ct));

name_extrmLow = ['LLW_NTDE_MLLW_',thresholds_txt{thres_ct}];

name_extrmHigh = ['HHW_NTDE_MLLW_',thresholds_txt{thres_ct}];

STATION(station_ct).(name_extrmLow) = tmp_extrmLow;

STATION(station_ct).(name_extrmHigh) = tmp_extrmHigh;

clear tmp_extrmLow tmp_extrmHigh name_extrmLow name_extrmHigh

end

clear thresholds thresholds_txt

%% -------------------------------------------

clear('diurnal_flag','sl_NTDE','HighTides','LowTides');

else

disp(strcat('No Hourly Tide Gauge Data during NTDE: ',STATION(station_ct).station_UHSLC));

STATION(station_ct).TideType = '';

STATION(station_ct).MHHW_NTDE = NaN;

STATION(station_ct).MHW_NTDE = NaN;

STATION(station_ct).DTL_NTDE = NaN;

STATION(station_ct).MTL_NTDE = NaN;

STATION(station_ct).MSL_NTDE = NaN;

STATION(station_ct).MLW_NTDE = NaN;

STATION(station_ct).MLLW_NTDE = NaN;

STATION(station_ct).GT_NTDE = NaN;

STATION(station_ct).MN_NTDE = NaN;

STATION(station_ct).DHQ_NTDE = NaN;

STATION(station_ct).DLQ_NTDE = NaN;

STATION(station_ct).HWI_NTDE = NaN;

STATION(station_ct).LWI_NTDE = NaN;

%STATION(station_ct).sl_NTDE_MLLW(:,1) = NaN;

STATION(station_ct).SLntde_extrmLow_90 = NaN;

STATION(station_ct).SLntde_extrmLow_95 = NaN;

STATION(station_ct).SLntde_extrmLow_99 = NaN;

STATION(station_ct).SLntde_extrmHigh_90 = NaN;

STATION(station_ct).SLntde_extrmHigh_95 = NaN;

STATION(station_ct).SLntde_extrmHigh_99 = NaN;

STATION(station_ct).LLW_NTDE_MLLW_90 = NaN;

STATION(station_ct).LLW_NTDE_MLLW_95 = NaN;

STATION(station_ct).LLW_NTDE_MLLW_99 = NaN;

STATION(station_ct).HHW_NTDE_MLLW_90 = NaN;

STATION(station_ct).HHW_NTDE_MLLW_95 = NaN;

STATION(station_ct).HHW_NTDE_MLLW_99 = NaN;

end

else

disp(strcat('No Hourly Tide Gauge Data: ',STATION(station_ct).station_UHSLC));

STATION(station_ct).TideType = '';

STATION(station_ct).MHHW_NTDE = NaN;

STATION(station_ct).MHW_NTDE = NaN;

STATION(station_ct).DTL_NTDE = NaN;

STATION(station_ct).MTL_NTDE = NaN;

STATION(station_ct).MSL_NTDE = NaN;

STATION(station_ct).MLW_NTDE = NaN;

STATION(station_ct).MLLW_NTDE = NaN;

STATION(station_ct).GT_NTDE = NaN;

STATION(station_ct).MN_NTDE = NaN;

STATION(station_ct).DHQ_NTDE = NaN;

STATION(station_ct).DLQ_NTDE = NaN;

STATION(station_ct).HWI_NTDE = NaN;

STATION(station_ct).LWI_NTDE = NaN;

%STATION(station_ct).sl_NTDE_MLLW(:,1) = NaN;

STATION(station_ct).SLntde_extrmLow_90 = NaN;

STATION(station_ct).SLntde_extrmLow_95 = NaN;

STATION(station_ct).SLntde_extrmLow_99 = NaN;

STATION(station_ct).SLntde_extrmHigh_90 = NaN;

STATION(station_ct).SLntde_extrmHigh_95 = NaN;

STATION(station_ct).SLntde_extrmHigh_99 = NaN;

STATION(station_ct).LLW_NTDE_MLLW_90 = NaN;

STATION(station_ct).LLW_NTDE_MLLW_95 = NaN;

STATION(station_ct).LLW_NTDE_MLLW_99 = NaN;

STATION(station_ct).HHW_NTDE_MLLW_90 = NaN;

STATION(station_ct).HHW_NTDE_MLLW_95 = NaN;

STATION(station_ct).HHW_NTDE_MLLW_99 = NaN;

end

end

%%==================================================================

## Tidal analysis

%% ========================================================================

%% Tide analysis and prediction using "UTide"

%%   Called by Scott's "UT_solve" function

% http://www.po.gso.uri.edu/~codiga/utide/utide.htm

% Parameters for UT_solv

%anal = 'annual'; % Analysis option (year-by-year)

anal = 'epoch'; % Analysis option (all years together)

%opt = 'stats'; % Full analysis power

opt = 'nostats'; % Less analysis power (but still good)

SAopt = 1; % Include Sa and Ssa

MinSNR = 2; % Same as utide default

plt = 2; % full timeseries plot

NodsatNone = 0; % 0 = include nodal cycle, 1 = omit nodal cycle

for station_ct = 1:size(uhidTxt,1)

if ~isnan(STATION(station_ct).latitude{1})  % Tide gauge exists

t = STATION(station_ct).t_NTDE(:,1);

y = STATION(station_ct).sl_NTDE(:,1);

% Done above

%     % Sort the time and sea level data by increasing time (6/28/24, address UH001 issue)

%     [t_sort, sort_index] = sort(t);  % Sort time and get the sorting index

%     y_sort = y(sort_index);          % Apply the sorting index to sea level data

%

%     % Find unique times and their indices

%     [t_unique, unique_index] = unique(t_sort, 'first');

%

%     % Keep only the unique times and corresponding sea level data

%     t = t_unique;

%     y = y_sort(unique_index);

%

%     % Clear temporary variables

%     clear t_sort y_sort sort_index unique_index

t_datenum = t + datenum(1800,1,1); % Convert to "datenum" format (days since Jan-1-0000)

y_noNaN = y(~isnan(y));

STATION(station_ct).nHr_NTDE = size(y_noNaN,1); % Number of hours available during NTDE

lat = STATION(station_ct).latitude{1};

if size(y_noNaN,1)>t_min

tic

disp(strcat('UT_solve: ',STATION(station_ct).station_UHSLC))

[ytide,results,yres] = UT_solve(t_datenum,y,lat,anal,opt,SAopt,MinSNR,plt,NodsatNone);

toc

STATION(station_ct).ytide = ytide;

STATION(station_ct).results = results;

STATION(station_ct).yres = yres;

else

disp(strcat('Time series too short for UT_solve: ',STATION(station_ct).station_UHSLC))

STATION(station_ct).ytide = NaN;

STATION(station_ct).results = NaN;

STATION(station_ct).yres = NaN;

end

else

disp(strcat('No Hourly Tide Gauge Data: ',STATION(station_ct).station_UHSLC));

STATION(station_ct).nHr_NTDE = 0;

STATION(station_ct).ytide = NaN;

STATION(station_ct).results = NaN;

STATION(station_ct).yres = NaN;

end

clear('t','t_datenum','y','y_noNaN','lat','ytide','results','yres');

end

%%==================================================================

%% Calculate Extremes (Highest & Lowest Predicted Astronomical Tides)

for station_ct = 1:size(uhidTxt,1)

MLLW_NTDE = STATION(station_ct).MLLW_NTDE;

t = STATION(station_ct).t_NTDE(:,1);

results = STATION(station_ct).results;

t_datenum = t + datenum(1800,1,1); % Convert to "datenum" format (days since Jan-1-0000)

if isfield(results,'coef') % Coefficients exist

disp(strcat('Calculating Extremes: ',STATION(station_ct).station_UHSLC));

% Ignore trend

results_NoTrend = results;

results_NoTrend.coef.slope = 0;

% ----------------------------

% Tide prediction (NTDE epoch)

tic

[ sl_fit_NTDE, ~ ] = ut_reconstr(t_datenum, results_NoTrend.coef);

toc

sl_fit_NTDE_MLLW = (sl_fit_NTDE(:,1) - MLLW_NTDE);

% Highest Astronomical Tide (HAT) & Time

[HAT_NTDE i_HAT_NTDE] = max(sl_fit_NTDE);

STATION(station_ct).HAT_NTDE = HAT_NTDE;

t_HAT_NTDE = STATION(station_ct).t_NTDE(i_HAT_NTDE,1);

STATION(station_ct).t_HAT_NTDE = datestr(t_HAT_NTDE+datenum(1800,1,1));

clear('HAT_NTDE','t_HAT_NTDE');

% Lowest Astronomical Tide (LAT) & Time

[LAT_NTDE i_LAT_NTDE] = min(sl_fit_NTDE);

STATION(station_ct).LAT_NTDE = LAT_NTDE;

t_LAT_NTDE = STATION(station_ct).t_NTDE(i_LAT_NTDE,1);

STATION(station_ct).t_LAT_NTDE = datestr(t_LAT_NTDE+datenum(1800,1,1));

clear('LAT_NTDE','t_LAT_NTDE');

% ----------------------------

% Find High and Low tides (NTDE epoch)

delta = 10^-6;

[HighTides_NTDE_MLLW, LowTides_NTDE_MLLW] = peakdet(((sl_fit_NTDE(:,1) - MLLW_NTDE)),delta);

% ----------------------------

% Find Extreme High and Low tides (NTDE epoch)

%% Legacy extremes (High and low tide predictions)

threshold = 5; % Top and bottom %

sl_fit_NTDE_MLLW_extrmLow = prctile(LowTides_NTDE_MLLW(:,2),threshold);

sl_fit_NTDE_MLLW_extrmHigh = prctile(HighTides_NTDE_MLLW(:,2),100-threshold);

STATION(station_ct).sl_fit_NTDE_MLLW = sl_fit_NTDE_MLLW;

STATION(station_ct).sl_fit_NTDE_MLLW_extrmLow = sl_fit_NTDE_MLLW_extrmLow;

STATION(station_ct).sl_fit_NTDE_MLLW_extrmHigh = sl_fit_NTDE_MLLW_extrmHigh;

% New: 90%, 95%, 99%

thresholds = [100-90, 100-95, 100-99];

thresholds_txt = {'90','95','99'};

for thres_ct = 1:size(thresholds,2)

tmp_extrmLow = prctile(LowTides_NTDE_MLLW(:,2),thresholds(thres_ct));

tmp_extrmHigh = prctile(HighTides_NTDE_MLLW(:,2),100-thresholds(thres_ct));

name_extrmLow = ['Tide_extrmLow_',thresholds_txt{thres_ct}];

name_extrmHigh = ['Tide_extrmHigh_',thresholds_txt{thres_ct}];

STATION(station_ct).(name_extrmLow) = tmp_extrmLow;

STATION(station_ct).(name_extrmHigh) = tmp_extrmHigh;

clear tmp_extrmLow tmp_extrmHigh name_extrmLow name_extrmHigh

end

clear thresholds thresholds_txt

else

disp(strcat('No Tide Coefficients: ',STATION(station_ct).station_UHSLC));

STATION(station_ct).sl_fit_NTDE_MLLW = NaN;

STATION(station_ct).sl_fit_NTDE_MLLW_extrmLow = NaN;

STATION(station_ct).sl_fit_NTDE_MLLW_extrmHigh = NaN;

STATION(station_ct).HAT_NTDE = NaN;

STATION(station_ct).t_HAT_NTDE = NaN;

STATION(station_ct).LAT_NTDE = NaN;

STATION(station_ct).t_LAT_NTDE = NaN;

% New: 90%, 95%, 99%

thresholds = [100-90, 100-95, 100-99];

thresholds_txt = {'90','95','99'};

for thres_ct = 1:size(thresholds,2)

name_extrmLow = ['Tide_extrmLow_',thresholds_txt{thres_ct}];

name_extrmHigh = ['Tide_extrmHigh_',thresholds_txt{thres_ct}];

STATION(station_ct).(name_extrmLow) = NaN;

STATION(station_ct).(name_extrmHigh) = NaN;

clear name_extrmLow name_extrmHigh

end

clear thresholds thresholds_txt

end

clear('t','results','sl_fit_NTDE','sl_fit_NTDE_MLLW','HighTides_NTDE_MLLW','LowTides_NTDE_MLLW',...

'sl_fit_NTDE_MLLW_extrmLow','sl_fit_NTDE_MLLW_extrmHigh');

end

%%==================================================================

%% ======================================================================

%% Tidal Prediction (Long Future Epoch)

%% ------------------------------------

disp('Calculating & Saving Tidal Prediction (long future epoch)');

%dir_path = ('/home/kaimoku/mwidlans/Data/SeaLevel/Tidal/');

dir_path = (['/home/kaimoku/mwidlans/Data/SeaLevel/Tidal/v',version,'/']);

preStartYr = start_NTDE;

%preStartYr = 2020;

preEndYr = 2030;

%for station_ct = 1:size(uhidTxt,1)

%for station_ct = tide_partial_start:size(uhidTxt,1)  % Station subgroup only

for station_ct = tide_partial_start:size(STATION,2)  % rev 12/2/2021

disp(STATION(station_ct).station_UHSLC);

results = STATION(station_ct).results;

if isfield(results,'coef') % Coefficients exist

% Ignore trend

results_NoTrend = results;

results_NoTrend.coef.slope = 0;

%       % HHMM

%       t_fit_HHMM = [datenum(preStartYr,1,1,0,0,0):(1/(24*60)):datenum(preEndYr+1,1,1,0,0,0)-(1/(24*60))]; % Minutes

%       disp(strcat('Calling "ut_reconstr" (HHMM): ',num2str(preStartYr),'-',num2str(preEndYr)));

%       [ sl_fit, ~ ] = ut_reconstr(t_fit_HHMM, results_NoTrend.coef);

%

%       STATION(station_ct).time_PRE_FUTURE(:,1) = t_fit_HHMM(1,:)' - datenum(1800,1,1); % Index(Hours since 1800), Values(Days or Hours since 1800)

%       STATION(station_ct).tide_PRE_FUTURE(:,1) = sl_fit(1,:)'; % Index(Hours since 1800), Values(Harmonic Tidal Prediction)

% HH

t_fit_HH = [datenum(preStartYr,1,1,0,0,0):(1/24):datenum(preEndYr+1,1,1,0,0,0)-(1/24)]; % Hours

disp(strcat('Calling "ut_reconstr" (HH): ',num2str(preStartYr),'-',num2str(preEndYr)));

[ sl_fit, ~ ] = ut_reconstr(t_fit_HH, results_NoTrend.coef);

STATION(station_ct).time_PRE_FUTURE(:,1) = t_fit_HH(1,:)' - datenum(1800,1,1); % Index(Hours since 1800), Values(Days or Hours since 1800)

STATION(station_ct).tide_PRE_FUTURE(:,1) = sl_fit(1,:)'; % Index(Hours since 1800), Values(Harmonic Tidal Prediction)

end

%% Save Tide Predictions as CSV (added 10/28/2024)

%csv_filename = strcat(dir_path,uhidTxt{station_ct},'_TidePrediction_hourly_mm_StationZero_',num2str(preStartYr),'_',num2str(preEndYr),'.csv');

idx_avail = find(~isnan(STATION(station_ct).t_NTDE(:,1)));

pred_epoch_start_txt = datestr(STATION(station_ct).t_NTDE(idx_avail(1),1)+datenum(1800,1,1),'dd-mmm-yyyy');

pred_epoch_end_txt = datestr(STATION(station_ct).t_NTDE(idx_avail(end),1)+datenum(1800,1,1),'dd-mmm-yyyy');

csv_filename = strcat(dir_path,uhidTxt{station_ct},'_TidePrediction_hourly_mm_StationZero_',num2str(preStartYr),'_',num2str(preEndYr),'_BasedOn_',pred_epoch_start_txt,'_',pred_epoch_end_txt,'.csv');

clear idx_avail pred_epoch_start_txt pred_epoch_end_txt

% Format

reference_datum = 0 - 0; % No conversion (Zero to Zero) Reference datum

time_zone_opt = 0; % GMT

units_opt = convert_ssh; % Convert from meters back to mm

time_data = datestr(STATION(station_ct).time_PRE_FUTURE + datenum(1800,1,1) + datenum(0,0,0,time_zone_opt,0,0),'dd-mmm-yyyy HH');

tide_data = (STATION(station_ct).tide_PRE_FUTURE - reference_datum) * units_opt;

if units_opt == 1000 % mm

% Round to nearest integer

tide_data = round(tide_data);

end

% Combine time and tide predictions into a table

TidePredictionTable = table(time_data, tide_data, 'VariableNames', {'Time_GMT', 'TidePrediction_mm'});

% Write to CSV

writetable(TidePredictionTable, csv_filename);

clear id_txt csv_filename time_data tide_data TidePredictionTable

end

%% ------------------------------------

%% ======================================================================

%% Tidal Prediction (Multi-years)

disp('Calculating Tidal Prediction');

% preStartYr = 2016;

% preEndYr = 2020;

% preStartYr = 2018;

% preEndYr = 2023;

% preStartYr = 2020;

% preEndYr = 2026;

preStartYr = 2023;

preEndYr = 2029;

for station_ct = 1:size(uhidTxt,1)

%for station_ct = tide_partial_start:size(uhidTxt,1)  % Station subgroup only

%for station_ct = tide_partial_set % Station subgroup only

disp(STATION(station_ct).station_UHSLC);

results = STATION(station_ct).results;

if isfield(results,'coef') % Coefficients exist

% Ignore trend

results_NoTrend = results;

results_NoTrend.coef.slope = 0;

t_fit_HHMM = [datenum(preStartYr,1,1,0,0,0):(1/(24*60)):datenum(preEndYr+1,1,1,0,0,0)-(1/(24*60))]; % Minutes

%t_fit_HH = [datenum(preStartYr,1,1,0,0,0):(1/24):datenum(preEndYr+1,1,1,0,0,0)-(1/24)]; % Hours

disp(strcat('Calling "ut_reconstr" (HH): ',num2str(preStartYr),'-',num2str(preEndYr)));

[ sl_fit, ~ ] = ut_reconstr(t_fit_HHMM, results_NoTrend.coef);

%     STATION(station_ct).time_PRE(:,1) = t_fit_HHMM(1,:)' - datenum(1800,1,1); % Index(Hours since 1800), Values(Days or Hours since 1800)

%     STATION(station_ct).tide_PRE(:,1) = sl_fit(1,:)'; % Index(Hours since 1800), Values(Harmonic Tidal Prediction)

clear time_PRE tide_PRE

time_PRE(:,1) = t_fit_HHMM(1,:)' - datenum(1800,1,1); % Index(Hours since 1800), Values(Days or Hours since 1800)

tide_PRE(:,1) = sl_fit(1,:)'; % Index(Hours since 1800), Values(Harmonic Tidal Prediction)

% ----------------------------

% Find High and Low tides

delta = 10^-6;

MLLW_NTDE = STATION(station_ct).MLLW_NTDE;

[HighTides, LowTides] = peakdet(((sl_fit- MLLW_NTDE)),delta); % Relative to MLLW

time_diff = 3*60; % Check for "false" tide at start (3hr)

if HighTides(1,1) < LowTides(1,1) % High Tide first

if abs(HighTides(1,1)-LowTides(1,1))<time_diff

HighTides_fix = HighTides(2:end,:);

clear('HighTides');

HighTides = HighTides_fix;

clear('HighTides_fix');

end

elseif LowTides(1,1) < HighTides(1,1) % Low Tide first

if abs(LowTides(1,1)-HighTides(1,1))<time_diff

LowTides_fix = LowTides(2:end,:);

clear('LowTides');

LowTides = LowTides_fix;

clear('LowTides_fix');

end

end

t_HighTides = t_fit_HHMM(HighTides(:,1));

t_LowTides = t_fit_HHMM(LowTides(:,1));

% Local Standard Time

time_zone = STATION(station_ct).time_zone;

if isempty(time_zone) % Set as GMT

time_zone = 0;

end

%     % Greenwich Mean Time

%     time_zone = 0;

if time_zone>0

time_zone_txt = ['Local Standard Time (LST = GMT +',num2str(time_zone),' hr)'];

time_zone_txt_abb = ['(LST = GMT +',num2str(time_zone),' hr)'];

elseif time_zone<0

time_zone_txt = ['Local Standard Time (LST = GMT ',num2str(time_zone),' hr)'];

time_zone_txt_abb = ['(LST = GMT ',num2str(time_zone),' hr)'];

else

time_zone_txt = ['Greenwich Mean Time'];

time_zone_txt_abb = ['(GMT)'];

end

% Convert Time

t_HighTides_FORMAT = datestr(t_HighTides + datenum(0,0,0,time_zone,0,0),'dd-mmm-yyyy HH:MM');

t_LowTides_FORMAT = datestr(t_LowTides + datenum(0,0,0,time_zone,0,0),'dd-mmm-yyyy HH:MM');

t_fit_HHMM_FORMAT = t_fit_HHMM + datenum(0,0,0,time_zone,0,0);

t_fit_HHMM_FORMAT_datestr = datestr(t_fit_HHMM + datenum(0,0,0,time_zone,0,0),'dd');

n_TIDE = size(t_fit_HHMM_FORMAT,2); % Number of Minutes

time_month_TIDE = (([1:n_TIDE])/1 + 0);     % construct time array

xlim_month_TIDE = [min(time_month_TIDE(1,1)), max(time_month_TIDE(1,end))];  % plotting range

zero_line_TIDE = zeros(n_TIDE,1);

% ----------------------------

% Make Tide Table

TideTable = cell(size(t_HighTides_FORMAT,1)+size(t_LowTides_FORMAT,1),2); % (Times,Tides)

TideTable(1:end,1:end) =cellstr('NaN');

if (t_HighTides(1)-t_LowTides(1)) <0 % High Tide first

TideTable(1:2:end-1,1)= cellstr(t_HighTides_FORMAT(1:size(TideTable(1:2:end-1,1)),:));

%TideTable(1:2:end-1,2) = cellstr(num2str(roundn(HighTides(1:size(TideTable(1:2:end-1,1)),2),0)));

TideTable(1:2:end-1,2) = cellstr(num2str(roundn(HighTides(1:size(TideTable(1:2:end-1,1)),2),-2))); % m

TideTable(2:2:end,1) = cellstr(t_LowTides_FORMAT(1:size(TideTable(1:2:end-1,1)),:));

%TideTable(2:2:end,2) = cellstr(num2str(roundn(LowTides(1:size(TideTable(1:2:end-1,1)),2),0)));

TideTable(2:2:end,2) = cellstr(num2str(roundn(LowTides(1:size(TideTable(1:2:end-1,1)),2),-2))); % m

else  % Low Tide first

TideTable(1:2:end-1,1) = cellstr(t_LowTides_FORMAT(1:size(TideTable(1:2:end-1,1)),:));

%TideTable(1:2:end-1,2) = cellstr(num2str(roundn(LowTides(1:size(TideTable(1:2:end-1,1)),2),0)));

TideTable(1:2:end-1,2) = cellstr(num2str(roundn(LowTides(1:size(TideTable(1:2:end-1,1)),2),-2))); % m

TideTable(2:2:end,1) = cellstr(t_HighTides_FORMAT(1:size(TideTable(2:2:end,1)),:));

%TideTable(2:2:end,2) = cellstr(num2str(roundn(HighTides(1:size(TideTable(2:2:end,1)),2),0)));

TideTable(2:2:end,2) = cellstr(num2str(roundn(HighTides(1:size(TideTable(2:2:end,1)),2),-2))); % m

end

% ----------------------------

% Flag extreme tides & Water Levels

ExtrmLow_NTDE_MLLW = STATION(station_ct).sl_fit_NTDE_MLLW_extrmLow;

ExtrmHigh_NTDE_MLLW = STATION(station_ct).sl_fit_NTDE_MLLW_extrmHigh;

for ct = 1:size(TideTable,1)

% Tides

if (str2num(cell2mat(TideTable(ct,2))) <= ExtrmLow_NTDE_MLLW)|(str2num(cell2mat(TideTable(ct,2))) >= ExtrmHigh_NTDE_MLLW)

TideTable(ct,2) = strcat(TideTable(ct,2),'*');

end

end

% ----------------------------

end

function [ytide,results,yres] = UT_solve(t,y,lat,anal,opt,SAopt,MinSNR,plt,NodsatNone)

% Function to automatically run UTide for a timeseries, either on an annual

% basis, or for the whole epoch

% Usage:    [ytide,coefs,yres] = UT_solve(t,y,latitude,analysisopt,fullstats,SAopt,MinSNR,plt,NodsatNone)

% Inputs:   t = timeseries time-stamp (datenum format)

%           y = sea-level elevations

%           latitude

%           analysisopt = either 'annual', or 'epoch'

%           fullstats = either 'stats' for the full analysis power (uses

%                       stats toolbox), or 'nostats' to use 'LinCI' and

%                       'OLS' options (no stats toolbox).

%           SAopt = 1 to include Sa and Ssa, or ) to omit (default)

%           MinSNR = Only include constituents with SNR >= MinSNR. Default MinSNR=10.

%           plt = 1 for annual timeseries plots or 2 for full timeseries

%                       plot

%           NodsatNone = 1 to omit nodal cycle

% Outputs:  ytide = timeseries of predicted tides

%           coefs = harmonic coefficients

%           yres = non-tidal residual, either 'annual', or 'epoch'

% Scott Stephens

% To do a 100-year prediction for tidal nomograph

% coef = ut_solv(t,y,[],lat,'auto'); coef.mean = 0; coef.slope = 0;

% tt = [datenum(1900,1,1):1/24:datenum(2000,1,1)]';

% iSA = find(abs(coef.aux.frq - 1.140741040611774e-004) == min(abs(coef.aux.frq - 1.140741040611774e-004))); % SA frequency

% iSSA = find(abs(coef.aux.frq - 2.281591089992679e-004) == min(abs(coef.aux.frq - 2.281591089992679e-004))); % SSA frequency

% cns = coef.name; cns([iSA iSSA],:) = []; % Remove SA and SSA constituents from tidal predictions

% ytide = ut_reconstr(t,coef,'cnstit',cns,'MinSNR',10);

if ~exist('anal','var')

anal = 'annual'; % By default do harmonics on annual basis

end

if ~exist('opt','var')

anal = 'stats'; % By default use the full power, which requires stats toolbox

end

if ~exist('SAopt','var')

SAopt = '0'; % By default omit Sa and Ssa

end

if ~exist('MinSNR','var')

MinSNR = '10'; % By default adopt 10, the utide default is 2

end

if ~exist('NodsatNone','var')

NodsatNone = 0;

end

paccept = 75; % Only resolve tidal harmonics if > paccept% of non-NaN data

results = [];

ytide = NaN.*ones(size(y));

yres = NaN.*ones(size(y));

M2 = [];

S2 = [];

N2 = [];

K1 = [];

SA = [];

SSA = [];

if strcmp(anal,'annual') % Undertake harmonic analysis on an annual basis

[yr,mm] = datevec(t);

for ayr = min(yr):max(yr)

it = find(t >= datenum(ayr,1,1) & t <= datenum(ayr+1,1,2)); % To resolve SA, get 366-days

if isempty(it)

M2 = [M2; NaN NaN];

N2 = [N2; NaN NaN];

S2 = [S2; NaN NaN];

K1 = [K1; NaN NaN];

SA = [SA; NaN NaN];

SSA = [SSA; NaN NaN];

eval(['results.coef.c' num2str(ayr) ' = NaN;'])

else

is = it(1); ie = it(end); % Start and finish times for the year

if t(ie)-t(is) < 366 % To resolve SA, get 366-days

if ayr < max(yr)

ie = find(t >= t(is) + 366,1,'first');

else

is = find(t <= t(ie) - 366,1,'last');

end

end

if sum(isnan(y(is:ie))) < ((100-paccept)/100)* length(y(is:ie))

if strcmp(opt,'nostats')

if NodsatNone == 1

coef = ut_solv(t(is:ie),y(is:ie),[],lat,'auto','LinCI','OLS','White','NodsatNone');

else

coef = ut_solv(t(is:ie),y(is:ie),[],lat,'auto','LinCI','OLS','White');

end

else

if NodsatNone == 1

coef = ut_solv(t(is:ie),y(is:ie),[],lat,'auto','NodsatNone');

else

coef = ut_solv(t(is:ie),y(is:ie),[],lat,'auto');

end

end

iM2 = find(abs(coef.aux.frq - 0.080511400663758) == min(abs(coef.aux.frq - 0.080511400663758)));

M2 = [M2; coef.A(iM2) coef.g(iM2)];

iN2 = find(abs(coef.aux.frq - 0.078999248716515) == min(abs(coef.aux.frq - 0.078999248716515)));

N2 = [N2; coef.A(iN2) coef.g(iN2)];

iS2 = find(abs(coef.aux.frq - 0.083333333333333) == min(abs(coef.aux.frq - 0.083333333333333)));

S2 = [S2; coef.A(iS2) coef.g(iS2)];

iK1 = find(abs(coef.aux.frq - 0.041780746220811) == min(abs(coef.aux.frq - 0.041780746220811)));

K1 = [K1; coef.A(iK1) coef.g(iK1)];

iSA = find(abs(coef.aux.frq - 0.000228159108289) == min(abs(coef.aux.frq - 0.000228159108289)));

SA = [SA; coef.A(iSA) coef.g(iSA)];

iSSA = find(abs(coef.aux.frq - 0.000114074104247) == min(abs(coef.aux.frq - 0.000114074104247)));

SSA = [SSA; coef.A(iSSA) coef.g(iSSA)];

eval(['results.coef.c' num2str(ayr) ' = coef;'])

else

M2 = [M2; NaN NaN];

N2 = [N2; NaN NaN];

S2 = [S2; NaN NaN];

K1 = [K1; NaN NaN];

SA = [SA; NaN NaN];

SSA = [SSA; NaN NaN];

eval(['results.coef.c' num2str(ayr) ' = NaN;'])

end

eval(['results.t.t' num2str(ayr) ' = t(is:ie);'])

eval(['results.y.y' num2str(ayr) ' = y(is:ie);'])

if exist('coef','var')

coef_reconstr = coef; coef_reconstr.mean = 0; coef_reconstr.slope = 0;

if SAopt == 0 % omit Sa and Ssa

iSA = find(abs(coef.aux.frq - 1.140741040611774e-004) == min(abs(coef.aux.frq - 1.140741040611774e-004))); % SA frequency

iSSA = find(abs(coef.aux.frq - 2.281591089992679e-004) == min(abs(coef.aux.frq - 2.281591089992679e-004))); % SSA frequency

cns = coef.name; cns([iSA iSSA],:) = []; % Remove SA ans SSA constituents from tidal predictions

yp = ut_reconstr(t(is:ie),coef_reconstr,'cnstit',cns,'MinSNR',MinSNR);

else

yp = ut_reconstr(t(is:ie),coef_reconstr,'MinSNR',MinSNR); % Will use previous constituents if not enough values in this year

end

eval(['results.ytide.ytide' num2str(ayr) ' = yp(1:ie-is+1);'])

%[ayr ie-is length(yp)]

ytide(is:ie) = yp(1:ie-is+1);

yres(is:ie) = y(is:ie)-yp(1:ie-is+1);

if exist('plt','var')

if plt == 1

figure(1), clf

plot(t(is:ie),y(is:ie),'k',t(is:ie),ytide(is:ie),'r',t(is:ie),yres(is:ie),'g')

title(['Year: ' num2str(ayr)])

%                 pause(1)

end

end

else

eval(['results.ytide.ytide' num2str(ayr) ' = NaN.*ones(length(1:ie-is+1),1);'])

ytide(is:ie) = NaN.*ones(length(1:ie-is+1),1);

yres(is:ie) = NaN.*ones(length(is:ie),1);

end

end

results.har.M2 = M2;

results.har.S2 = S2;

results.har.N2 = N2;

results.har.K1 = K1;

results.har.SA = SA;

results.har.SSA = SSA;

results.har.year = min(yr):max(yr);

end

else % Undertake analysis on the whole timeseries at once

if strcmp(opt,'nostats')

if NodsatNone == 1

coef = ut_solv(t,y,[],lat,'auto','LinCI','OLS','White','NodsatNone');

else

coef = ut_solv(t,y,[],lat,'auto','LinCI','OLS','White');

end

else

if NodsatNone == 1

coef = ut_solv(t,y,[],lat,'auto','NodsatNone');

else

coef = ut_solv(t,y,[],lat,'auto');

end

end

results.coef = coef;

coef_reconstr = coef; coef_reconstr.mean = 0; coef_reconstr.slope = 0;% Don't include mean or slope in the tidal prediction

if SAopt == 0 % omit Sa and Ssa

iSA = find(abs(coef.aux.frq - 1.140741040611774e-004) == min(abs(coef.aux.frq - 1.140741040611774e-004))); % SA frequency

iSSA = find(abs(coef.aux.frq - 2.281591089992679e-004) == min(abs(coef.aux.frq - 2.281591089992679e-004))); % SSA frequency

cns = coef.name; cns([iSA iSSA],:) = []; % Remove SA and SSA constituents from tidal predictions

ytide = ut_reconstr(t,coef_reconstr,'cnstit',cns,'MinSNR',MinSNR);

else

ytide = ut_reconstr(t,coef_reconstr,'MinSNR',MinSNR);

end

yres = y-ytide;

end

if exist('plt','var')

if plt == 2

figure(1), clf

plot(t,y,'k',t,ytide,'r')

end

end